# Appendix: Roadway Block Variables Documentation

# Spatial Structure

Road segments are lines, tracts are polygons.

## Context

This study integrates roadway-level data (line geometries) with census tract-level data (polygons).Each road segment is spatially joined to a tract using its geometry.

## Variables

Census tract = polygon

GEOID = tract identifier

Road segment = line (LineString)

# 2. Identification Variables

## Context

These variables uniquely identify road segments but do not describe roadway characteristics.

## Variables

ROUTEID

Unique segment identifier

Used for joins and validation

❌ Not used in modeling

SHAPELEN

Segment length

⚠️ Observed as invalid (constant 0)

❌ Excluded

# 3. Traffic Exposure

AADT and truck composition variables.

AADT is modeled, not directly observed→ May introduce estimation bias

## Context

Traffic exposure is a primary driver of crash risk. Higher traffic volumes generally lead to more crash opportunities.

## Variables

AADT

Average daily traffic volume

✔️ Core exposure variable

AADT_COMBINATION

Heavy trucks (tractor-trailers)

AADT_SINGLE_UNIT

Medium trucks (delivery vehicles)

# 4. Road Geometry

Lane counts and widths.

## Context

Roadway geometry determines how traffic flows and how road users interact.

## Variables

TOTALTRAVELLANES

TOTALTRAVELLANEWIDTH (feet)

TOTALCROSSSECTIONWIDTH (feet)

INBOUND / OUTBOUND lanes

Higher speeds

More complex interactions

Increased crash risk

# 🚗 5. Parking Infrastructure

## Context

On-street parking changes traffic behavior and visibility conditions.

## Variables

TOTALPARKINGLANES

TOTALPARKINGLANEWIDTH (feet)

Obstruct visibility

Increase turning conflicts

Create dooring risks

# 6. Bike & Buffer

Bike lanes and physical buffers.

## Context

Bike infrastructure reflects multimodal design and affects safety for cyclists.

## Variables

TOTALBIKELANES

TOTALBIKELANEWIDTH (feet)

Cyclist safety

Mode separation

Conflict points

## Context

Buffers provide physical separation between traffic modes.

## Variables

TOTALRAISEDBUFFERS

TOTALRAISEDBUFFERWIDTH (feet)

Reduce collision probability

Improve safety perception

Separate vulnerable users

# Pavement

PCI and IRI measures.

## Context

Road surface quality affects vehicle control and crash likelihood.

## Variables

PCI_SCORE = Overall pavement condition rating (0–100)

Impact on safety:

- Lower PCI →

  - Reduced vehicle control

  - Increased braking distance

  - Higher likelihood of crashes

Note:
PCI is inspection-based, not continuous
→ It is not real-time data and may have a time lag

IRI = Road surface roughness indicator (commonly measured in m/km, not mph)

- Lower values indicate better ride quality

Note:
IRI is only collected on a subset of roads (primarily federal roads)
→ Data coverage is incomplete (important limitation)

SURFACE_TYPE = Road pavement material type (coded variable)

Common types include:

- Asphalt ✔️ (most common)

- Concrete

- Brick

- Gravel

Increase braking distance

Reduce vehicle stability

Contribute to crashes

# Traffic Calming

Vertical deflection features.

## Context

Traffic calming measures are designed to reduce vehicle speed.

## Variables

VERTICAL_DEFLECTION

Speed bump

Speed hump

Raised table

Speed (mph)

Crash severity

# Sidewalks

Sidewalk width and material.

## Context

Sidewalks represent pedestrian infrastructure and walkability.

## Variables

SIDEWALK_IB_WIDTH (feet)

SIDEWALK_OB_WIDTH (feet)

SIDEWALK_IB_PAVTYPE

SIDEWALK_OB_PAVTYPE

Pedestrian safety

Exposure to traffic

Accessibility

# Bus Lanes

Inbound vs outbound lanes.

- Bus lane variables are sparse→ May underrepresent transit infrastructure

## Context

Bus lanes indicate transit-priority infrastructure.

## Variables

BUSLANE_INBOUND

BUSLANE_OUTBOUND

Change lane allocation

Affect traffic flow

Introduce interaction with buses

# 🚦 12. Speed Limits

## Context

Speed is one of the strongest predictors of crash severity.

## Variables

SPEEDLIMITS_OB (mph)

SPEEDLIMITS_IB (not available)

Increase injury severity

Reduce reaction time

# FHWA Functional Class

National classification.

## Context

Functional class describes the role of a road in the network.

## Variables

FHWAFUNCTIONALCLASS

1 – Interstate

2 - Principal Arterial - Other Freeways or Expressways

3 - Principal Arterial – Other

4 - Minor Arterial

5 - Major Collector

6 - Minor Collector

7 – Local

Traffic volumes

Speeds

Risk profiles

# 14. DC Functional Class

## Variables

DCFUNCTIONALCLASS

Local DC classification

11 – Interstate

12 – Other Freeway and Expressway

14 – Principal Arterial

16 – Minor Arterial

17 – Major Collector

18 – Minor Collector

19 – Local

## Context

DC classification provides local refinement of roadway hierarchy.

# 📌 16. Data Limitations and Coverage

## Context

This dataset contains several known limitations related to data completeness, coverage, and measurement methodology.

## Key limitations